# Second Project Report

Which Model When: Comparing Three Ways to Predict Where a Car Will Drive

Members: Member A, Member B, Member C, Member D · Code: https://github.com/<username>/<repo>

# 1. The data we use

We use nuPlan mini, a public self-driving dataset: 64 driving recordings from four cities (Las Vegas, Singapore, Boston, Pittsburgh), at 10 frames per second.

We cut the recordings into short examples. A model sees the last 2 seconds of one car's motion plus up to 10 nearby cars, and predicts where that car will be over the next 8 seconds (80 positions). Each example is rotated so the car starts at the origin facing forward.

After removing cars that barely moved, 297,875 examples remain. We split them by whole recording — never by frame, so the same trip cannot appear in training and testing — and kept all four cities in every part: 175,522 for training, 61,327 for training the model selector, and 61,026 held out for final testing.

# 2. The three models we compare

•  A — Regression: one direct guess. Very fast.

•  B — Diffusion: refines random noise in 15 small steps into a trajectory; can give

several plausible futures.

•  C — Flow matching: a cheaper variant of the same idea, needing only 4 steps.

All three share the same input encoder and the same 3.69 M-parameter trajectory network; only the training rule and the generation procedure differ. Otherwise a win could simply mean "the bigger network won". All three also obey one fixed physical rule: the path must be smooth, realistic, and must start exactly at the car's current position.

# 3. What we have so far

Results on the 61,026 examples no model ever saw (three random runs averaged; error is the distance to the true position, averaged over 8 s, best of 6 candidates):

| model | network calls | average position error | error at 8 s | time per prediction |
| --- | --- | --- | --- | --- |
| baseline: keep current speed | 1 | 5.94 m | 16.88 m | — |
| A regression | 1 | 2.82 m | 8.25 m | 6 ms |
| C flow matching | 24 | 1.83 m | 4.83 m | 19 ms |
| B diffusion | 90 | 1.74 m | 4.53 m | 60 ms |

1. Both generative models clearly beat regression (about 1.5×); all three beat the

"keep current speed" baseline by 3× or more.

2. Our second research question is answered: flow matching is nearly as good — only

5 % worse than diffusion at about a quarter of the computation. Giving it more than 4 steps changes nothing.

3. We verified the implementation rather than assuming it works: given a perfect target,

generation reproduces it exactly (error 0.000), while a deliberately broken control correctly fails.

Model selector (in progress). The goal is to pick the *cheapest* model that is still good enough. Perfect per-case selection would cut the error by 22 %, but that is misleading: the minimum of several noisy measurements always looks better than reality. Choosing the model on half of each model's candidates and scoring on the other half leaves a real gain of only 4.7 %. Our learned selector recovers 1–2 % — about a third of what is available.

# 4. Problems we ran into

Data. Positions are stored as huge global coordinates; converting them to local size in low precision shifted every position by up to 25 cm. Recorded positions also jitter between frames, so even the ground truth looks physically impossible when differentiated, and our plausibility check had to be made relative to the data.

Models. Our first version predicted the *noise* instead of the trajectory, which amplified small errors into unusable output. Our first comparison was unfair (one model had 8× fewer parameters) and was corrected. The standard trick for varied predictions — training on the best of many samples — fails badly here, because it leaves the generator undefined for most random inputs.

Known weaknesses. Predictions are too smooth, so turns are underestimated. The 6 candidate futures cover about 2 m sideways where the truth spreads about 20 m. Sharp turns at high speed are usually missed, and there is no map information yet.

# 5. Next steps

1. Add road and lane geometry — the most likely fix for missed turns.

2. Improve the selector with better input features.

3. Replace the failed "best of many samples" idea with one that works for step-by-step

generation.

4. Repeat with more random runs, broken down by scenario type.