# Second Project Report

Which Model When: Comparing Three Ways to Predict Where a Car Will Drive

Members: Member A, Member B, Member C, Member D · Code: https://github.com/Parick100/nuplan-model-selector

# 1. The data we use

nuPlan mini, a public self-driving dataset: 64 driving recordings from four cities (Las Vegas, Singapore, Boston, Pittsburgh) at 10 frames per second.

We cut the recordings into examples. A model sees the last 2 seconds of one car's motion plus up to 10 nearby cars, and predicts that car's position over the next 8 seconds (80 points). Each example is rotated so the car starts at the origin facing forward.

After removing cars that barely moved, 297,875 examples remain, split by whole recording (never by frame, so the same trip cannot appear in both training and testing) with all four cities in every part: 175,522 for training, 61,327 for training the selector, 61,026 held out for testing.

# 2. The three models we compare

•  A — Regression: one direct guess. Very fast.

•  B — Diffusion: refines random noise in 15 small steps into a trajectory; gives

several plausible futures.

•  C — Flow matching: a cheaper variant needing only 4 steps.

All three share the same encoder and the same 3.69 M-parameter trajectory network; only the training rule and the generation procedure differ — otherwise a win could simply mean "the bigger network won". All three also obey one fixed physical rule: the path must be smooth, realistic and start exactly at the car's current position.

# 3. What we have so far

Evaluated on 4,096 randomly chosen scenes from the 61,026 that no model ever saw (three random runs averaged; error = distance to the true position, averaged over 8 s, best of 6 candidates). This is open-loop: we score against the recorded future, not a full driving simulation.

| model | network calls | average position error | error at 8 s | time per prediction |
| --- | --- | --- | --- | --- |
| baseline: keep current speed | 1 | 5.94 m | 16.88 m | — |
| A regression | 1 | 2.82 m | 8.25 m | 6 ms |
| C flow matching | 24 | 1.83 m | 4.83 m | 19 ms |
| B diffusion | 90 | 1.74 m | 4.53 m | 60 ms |

1. Both generative models clearly beat regression (about 1.5×), and all three beat

the constant-speed baseline by 3× or more.

2. For the models we trained here, flow matching is nearly as good as diffusion —

5 % worse at about a quarter of the computation. That is a statement about *our* trained pair, not about the two methods in general; more than 4 steps does not help.

3. We verified the implementation rather than assuming: given a perfect target,

generation reproduces it exactly (error 0.000), while a broken control fails.

Model selector (in progress). The goal is to pick the *cheapest* model that is still good enough. The ceiling first: assigning every scene its best model perfectly would cut the error from 1.72 m to 1.35 m (21 %). Our two trained selectors recover 5–12 % of that ceiling. The limit is not the ceiling but our ability to predict it: whether flow matching or diffusion wins on a given scene is essentially unpredictable from scene features (correlation −0.03) and is mostly sampling noise anyway, while regression-versus-generative is a real per-scene difference (split-half reliability 0.60) that is still only weakly predictable (0.15). The lever is better input features, i.e. map and lane geometry.

# 4. Problems we ran into

Data. Positions are stored as huge global coordinates; converting them to local size in low precision shifted every position by up to 25 cm. Recorded positions also jitter between frames, so even the ground truth looks physically impossible once differentiated, and our plausibility check had to be made relative to the data.

Models. Our first version predicted the *noise* instead of the trajectory, which amplified small errors into unusable output. Our first comparison was unfair (one model had 8× fewer parameters) and was corrected. The standard trick for varied predictions — training on the best of many samples — fails here: it leaves the generator undefined for most random inputs.

Known weaknesses. Predictions are too smooth, so turns are underestimated. The 6 candidates cover about 2 m sideways where the truth spreads about 20 m. Sharp turns at high speed are missed, and there is no map information yet.

# 5. Next steps

1. Add road and lane geometry — the most likely fix for missed turns.

2. Improve the selector with those features.

3. Replace the failed "best of many samples" idea with one that suits step-by-step

generation.

4. Repeat with more random runs, broken down by scenario type.